WEEK 2 – FIRMWARE DEVELOPMENT AND IMPLEMENTATION

Timer Interrupt Based LED Controller

# 1. Objective

The objective of this task is to design and implement a simple firmware application using Embedded C. The project demonstrates timer-based interrupt handling, GPIO control, firmware organization, debugging, documentation, and basic testing. Since dedicated hardware is not required, the timer and LED are simulated using software variables and functions.

# 2. Project Overview

The Timer Interrupt Based LED Controller is a small embedded firmware application in which a simulated hardware timer generates a periodic event. When the timer reaches its configured period, a timer interrupt is generated. The interrupt service routine toggles the LED state. This demonstrates the basic principle of interrupt-driven firmware without depending on a specific microcontroller.

# 3. Firmware Architecture

System Initialization → GPIO initialization and timer initialization.

Timer Module → Maintains the simulated timer tick counter.

Interrupt Module → Detects the timer event and calls the timer ISR.

GPIO/LED Module → Controls the simulated LED state.

Main Application → Continuously executes the firmware loop and timer ticks.

# 4. Program Flow

1. Start the application.
2. Initialize the simulated GPIO and set the LED to OFF.
3. Initialize the timer and clear the interrupt flag.
4. Execute the main loop.
5. Increment the timer on every simulated tick.
6. When the timer reaches 1000 ticks, generate an interrupt.
7. Execute Timer_ISR().
8. Toggle the LED state.
9. Reset the timer counter and continue the loop.

# 5. Key Functions

| Function | Purpose |
| --- | --- |
| GPIO_Init() | Initializes the simulated LED GPIO. |
| Timer_Init() | Initializes the timer counter and interrupt flag. |
| LED_Set() | Sets the LED state to ON or OFF. |
| LED_Toggle() | Toggles the current LED state. |
| Timer_Tick() | Simulates one timer tick and checks for interrupt generation. |
| Timer_ISR() | Handles the simulated timer interrupt. |
| System_Init() | Calls the required initialization functions. |
| main() | Runs the firmware simulation. |

# 6. Embedded C Source Code

/*

* Week 2: Firmware Development and Implementation

* Project: Timer Interrupt Based LED Controller

*

* Description:

* This is a portable Embedded-C simulation of a timer-interrupt-driven

* LED controller. A simulated timer generates a periodic interrupt.

* The interrupt service routine toggles the LED state.

*

* Note:

* The hardware registers are simulated with variables so that the

* firmware logic can be compiled and tested on a normal computer.

*/

#include <stdio.h>

#include <stdbool.h>

#include <stdint.h>

/* -------------------- Simulated Hardware -------------------- */

volatile bool LED_STATE = false;       // Simulated GPIO output

volatile uint32_t timer_ticks = 0;     // Simulated timer tick counter

volatile bool timer_interrupt = false; // Simulated interrupt flag

#define TIMER_PERIOD_TICKS 1000U

/* -------------------- Function Prototypes -------------------- */

void GPIO_Init(void);

void Timer_Init(void);

void LED_Set(bool state);

void LED_Toggle(void);

void Timer_ISR(void);

void Timer_Tick(void);

void System_Init(void);

/* -------------------- GPIO Functions -------------------- */

void GPIO_Init(void)

{

LED_STATE = false;

printf("GPIO initialized: LED = OFF\n");

}

void LED_Set(bool state)

{

LED_STATE = state;

printf("LED = %s\n", LED_STATE ? "ON" : "OFF");

}

void LED_Toggle(void)

{

LED_Set(!LED_STATE);

}

/* -------------------- Timer Functions -------------------- */

void Timer_Init(void)

{

timer_ticks = 0;

timer_interrupt = false;

printf("Timer initialized: period = %u ticks\n", TIMER_PERIOD_TICKS);

}

/*

* Timer interrupt service routine.

* In a real microcontroller, this function would be called by

* the processor when the hardware timer reaches its configured period.

*/

void Timer_ISR(void)

{

timer_interrupt = false;

LED_Toggle();

}

/*

* Simulate one timer tick.

*/

void Timer_Tick(void)

{

timer_ticks++;

if (timer_ticks >= TIMER_PERIOD_TICKS)

{

timer_ticks = 0;

timer_interrupt = true;

}

/* Process the simulated interrupt immediately. */

if (timer_interrupt)

{

Timer_ISR();

}

}

/* -------------------- System Initialization -------------------- */

void System_Init(void)

{

GPIO_Init();

Timer_Init();

}

/* -------------------- Main Application -------------------- */

int main(void)

{

uint32_t total_ticks = 5000U;

printf("========================================\n");

printf(" Timer Interrupt Based LED Controller\n");

printf("========================================\n");

System_Init();

/*

* Main loop simulation.

* Each iteration represents one timer tick.

*/

for (uint32_t i = 0; i < total_ticks; i++)

{

Timer_Tick();

}

printf("Simulation completed.\n");

printf("Final LED state: %s\n", LED_STATE ? "ON" : "OFF");

return 0;

}

# 7. Testing and Verification

The firmware was tested using a software simulation. The simulation was configured with a timer period of 1000 ticks and executed for 5000 ticks. Therefore, five timer interrupt events are expected. Every interrupt calls the Timer_ISR() routine, which toggles the LED. The console output is used to verify initialization, interrupt execution, LED transitions, and the final state.

Expected verification points:

GPIO initialization reports that the LED starts in the OFF state.

Timer initialization reports a period of 1000 ticks.

The LED changes state whenever the timer interrupt occurs.

Five interrupt events occur during the 5000-tick simulation.

The program completes without a runtime error.

# 8. Compilation and Simulation Instructions

The source file can be compiled using GCC. Example command:
gcc timer_led_controller.c -o timer_led_controller

After compilation, run the generated executable. On Linux/macOS use ./timer_led_controller. On Windows use timer_led_controller.exe. The console output displays the simulated LED transitions.

# 9. README

WEEK 2 - FIRMWARE DEVELOPMENT AND IMPLEMENTATION
Project: Timer Interrupt Based LED Controller

1. OVERVIEW
This project demonstrates a simple interrupt-driven firmware application.
A simulated timer generates a periodic interrupt and the interrupt service
routine toggles a simulated LED. Hardware registers are represented by
variables so that the program can be compiled on a normal computer.

2. MAIN FUNCTIONS
- GPIO_Init()  : Initializes the simulated LED GPIO.
- Timer_Init() : Initializes the simulated timer and interrupt flag.
- LED_Set()    : Sets the LED to ON or OFF.
- LED_Toggle() : Changes the current LED state.
- Timer_Tick() : Simulates one timer tick and detects timer overflow.
- Timer_ISR()  : Simulated timer interrupt service routine.
- System_Init(): Performs system initialization.
- main()       : Runs the firmware simulation.

3. HOW TO COMPILE
Using GCC:
    gcc timer_led_controller.c -o timer_led_controller

4. HOW TO RUN
Linux/macOS:
    ./timer_led_controller

Windows:
    timer_led_controller.exe

5. EXPECTED OUTPUT
The program prints the initialization messages and then toggles the LED
after every TIMER_PERIOD_TICKS timer ticks. The output should contain
alternating LED ON and LED OFF messages.

6. TESTING
The simulation runs for 5000 timer ticks with a timer period of 1000 ticks.
Therefore, the interrupt service routine is expected to execute five times.
The LED changes state each time the simulated interrupt occurs.

7. NOTE
This is a hardware-independent simulation. For deployment on a specific
microcontroller, the GPIO and timer variables/functions should be replaced
with the corresponding MCU register configuration and interrupt vector.

# 10. Learning Outcomes

Improved understanding of Embedded C program structure.

Understood the role of timer interrupts in embedded systems.

Practiced GPIO-style LED control logic.

Learned to separate initialization, driver logic, interrupt handling, and main application code.

Practiced basic firmware testing and debugging using console output.

Improved code documentation through inline comments and a README.

# 11. Conclusion

The task successfully demonstrates the development of a simple interrupt-driven firmware application. The timer-based LED controller provides a clear example of how periodic hardware events can be handled by an interrupt service routine. The modular source code, comments, README, and testing procedure provide a structured foundation that can later be adapted to a real microcontroller by replacing the simulated timer and GPIO operations with device-specific registers.